| Package Title: |  | Number: |  |
| --- | --- | --- | --- |
| Revision: |  |  |  |
| Package No: |  | Type: |  |

| Title: |  |  |
| --- | --- | --- |
| Manufacturer: |  | Location: |
| Agent: |  | Discipline: |
| Contractor: |  |  |

Specification:

Contract Ref:

BOQ Ref:

Other Ref:

### Description:

Mô tả:

NSF contractor would like to submit the document "Vlan IP Planing " / Nhà thầu NFS xin đệ trình lại tài liệu "Kế hoạch Vlan IP"

### Enclosures:

Đính kèm:

1)

2)

3)

| SUBMITTAL COMMENTS/ Nhận xét về Hồ sơ Đệ trình |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REV | RECEIVED BY EMPLOYER REP | COMMENTS/ Nhận Xét | STATUS/ Tình trạng |  |  |  |  | REVIEWED BY/ Xem xét bởi |  |
|  |  |  | A | B | C | D | E |  |  |
| 00 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | PEPSICO |
|  |  |  |  |  |  |  |  |  | NSP |
| 01 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | PEPSICO |
|  |  |  |  |  |  |  |  |  | NSP |
| 02 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | PEPSICO |
|  |  |  |  |  |  |  |  |  | NSP |
| 03 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | PEPSICO |
|  |  |  |  |  |  |  |  |  | NSP |

### EMPLOYER COMMENTS (IF ANY):

Chủ đầu tư Nhận xét (Nếu có):

### STATUS CODES:

- Approved

Phê duyệt

- Approved as Noted

Phê duyệt như Nhận xét

- Revise and Resubmit

Sửa và Đệ trình lại

- Reject

Từ chối

- Reference Only

Chỉ Tham khảo

CONTRACTOR DETAILS/ Thông tin Nhà thầu

### Prepared By:

Chuẩn bị bởi:

### Title:

Chức vụ:

130002

January 2019 R00

### Reviewed By:

Xem xét bởi:

### Title:

PAGE | 1

DWIGHT SCHOOL VLAN IP PLANNING

# Summary

This document will provide the IP address for all Guest Data and Wi-Fi, Data, Tel, AV, CCTV, ACS. This document also provides the IP and Hostname of devices for management.

# IP address for DATA network

When a PC plug in to LAN outlet at Classroom and Office, it will receive an IP address from DHCP server. DHCP server provides IP address pool.

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Office & Classroom | 8 | 10.0.8.0 | 21 | 10.0.8.1 | 10.0.9.1 | 10.0.15.254 | Dynamic |
|  |  |  |  |  | 10.0.8.2 | 10.0.8.254 | Static |

# IP address for Wi-Fi network

- The dynamic addresses will be assigned from 50 to the last address of the network class, while addresses from 2 to 50 will be static addresses. The .1 address will be the gateway.

| System | Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSID | DHS_Classroom | 24 | 10.0.24.0 | 21 | 10.0.24.1 | 10.0.24.50 | 10.0.31.254 |
|  | DHS_Staff | 32 | 10.0.32.0 | 21 | 10.0.32.1 | 10.0.32.50 | 10.0.39.254 |
|  | DHS_Guest | 40 | 10.0.40.0 | 21 | 10.0.40.1 | 10.0.40.50 | 10.0.47.254 |
|  | DHS_BYOD | 48 | 10.0.48.0 | 21 | 10.0.48.1 | 10.0.48.50 | 10.0.55.254 |

- When a Laptop, Smart Phone, Mobility Devices connection to a Guest SSID will block all network traffic from guest users accessing the internal network.

# IP address for AV system

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AV | 56 | 10.0.56.0 | 23 | 10.0.56.1 | 10.0.56.50 | 10.0.57.254 | Dynamic |
|  |  |  |  |  | 10.0.56.2 | 10.0.56.49 | Static |

# IP address for IP Phone system (TEL)

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TEL | 16 | 10.0.16.0 | 23 | 10.0.16.1 | 10.0.16.50 | 10.0.17.254 | Dynamic |
|  |  |  |  |  | 10.0.16.2 | 10.0.16.49 | Static |

# Management VLAN and Hostname

  - The hostname:

    - The device name should be: <Location Name>_YY

| AP | WI-FI Access Point |
| --- | --- |
| L | LAN Outlet |
| T | Tel Outlet |
| TV | TV Outlet |
| MC | Master Clock |
| R1_AC1 | Switch |

YY: The sequential number of devices of floor

## For example:

The Wi-Fi Access Point number 03 at 2 floor: WIFI-2-3

    - The switch name should be: <Name Devices >_YY

  - Managerment Network IP address for administrator:

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MGMT | 999 | 10.0.0.0 | 23 | 10.0.0.1 | 10.0.0.50 | 10.0.1.254 | Dynamic |
|  |  |  |  |  | 10.0.0.2 | 10.0.0.49 | Static |

# IP address for CCTV system

- This information is used to provide details about the network subnet and the Gateway address configured on the Core Switch, including whether DHCP is enabled. If dynamic IP allocation via DHCP is not used, please leave it blank, and all addresses will be static.

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CCTV | 58 | 10.0.58.0 | 23 | 10.0.58.1 | 10.0.58.50 | 10.0.59.254 | Dynamic |
|  |  |  |  |  | 10.0.58.2 | 10.0.58.49 | Static |

# IP address for ACS system

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ACS | 60 | 10.0.60.0 | 23 | 10.0.60.1 | 10.0.60.50 | 10.0.61.254 | Dynamic |
|  |  |  |  |  | 10.0.60.2 | 10.0.60.49 | Static |

# IP address for PAVA

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PAVA | 62 | 10.0.62.0 | 23 | 10.0.62.1 | 10.0.62.50 | 10.0.63.254 | Dynamic |
|  |  |  |  |  | 10.0.62.2 | 10.0.62.49 | Static |

# IP address for Carparking

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Carparking | 64 | 10.0.64.0 | 23 | 10.0.64.1 | 10.0.64.50 | 10.0.65.254 | Dynamic |
|  |  |  |  |  | 10.0.64.2 | 10.0.64.49 | Static |

# IP address for GTS

| Location | VLAN ID | Network | Prefix-length | Gateway | Start IP | End IP | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GTS | 66 | 10.0.66.0 | 23 | 10.0.66.1 | 10.0.66.50 | 10.0.67.254 | Dynamic |
|  |  |  |  |  | 10.0.66.2 | 10.0.66.49 | Static |

# Devices IP Address:

Please fill in the information below, and we will proceed accordingly.

| Location | Name Device | IP Address | S/N | Gateway | DHCP | DHCP Server | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R4H17A | AP-575-1 | 10.0.66.X | FOC… | 10.0.66.1 | Yes | 10.0.66.1 | AP |
| JL662A |  |  |  |  |  |  |  |
| JL679A |  |  |  |  |  |  |  |
| 500618-004 |  |  |  |  |  |  |  |
| 500618-004 |  |  |  |  |  |  |  |
| OS6360-PH24 |  |  |  |  |  |  |  |
| OS6860N-U28 |  |  |  |  |  |  |  |
| OXO Connect Large 150W |  |  |  |  |  |  |  |
| OS6360-P10 |  |  |  |  |  |  |  |
| R7J27A |  |  |  |  |  |  |  |
| JL658A |  |  |  |  |  |  |  |